DAFFODIL INTERNATIONAL UNIVERSITY

DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING

FYDP (Phase-I) Evaluation Report

reporting period- spring 2026

Project Identification:

| I. Project Title | Edge AI-Powered Acoustic Surveillance System for Forest Protection Using TinyML |  |
| --- | --- | --- |
| II. Group Members | - Name: Student ID: - Name: Student ID: |  |
| III. Supervisor | Name: Designation: |  |
| IV. Co-Supervisor | Name: Designation: |  |
| V. Submission Date: |  |  |
| VI. Certificate: | “This is to certify that the final year design project work until Phase-I evaluation held on ___________________, titled as stated in Sec. I, executed by the students’ group mentioned in Sec. II, have been found satisfactory and every section of this report is reflecting the same.” | (Signature of Supervisor & date) |

Project Insights

| Thematic Area(s): [Just click the check box] | Artificial Intelligence and Machine Learning | ☐ |
| --- | --- | --- |
|  | Deep Learning | ☐ |
|  | Health Informatics | ☐ |
|  | Cybersecurity | ☐ |
|  | Software Engineering and Development | ☐ |
|  | Blockchain Technology | ☐ |
|  | Internet of Things (IoT) | ☐ |
|  | Computer Networks | ☐ |
|  | Computer Vision | ☐ |
|  | Natural Language Processing (NLP) | ☐ |
|  | Robotics | ☐ |
|  | Game Development | ☐ |
|  | Cloud Computing | ☐ |
|  | Image Processing | ☐ |
|  | Others (please specify): |  |
| Software packages, tools, and programming languages | Python, TensorFlow/Keras, TensorFlow Lite Micro, Edge Impulse Studio, Arduino IDE (C/C++), OpenPyXL, NumPy, Librosa, Matplotlib, Scikit-learn |  |

CO Description for FYDP-Phase-I

| CO | CO Descriptions | PO |
| --- | --- | --- |
| CO4 | Perform economic evaluation, cost estimation, and apply suitable project management procedures throughout the FYDP lifecycle in the context of developing the "Edge AI-Powered Acoustic Surveillance System for Forest Protection" project. | PO11 |
| CO6 | Select and apply appropriate methodologies, resources, and contemporary engineering/IT tools for prediction, modeling, and solving complex engineering processes for the "Edge AI-Powered Acoustic Surveillance System for Forest Protection" project. | PO5 |
| CO7 | Assess societal, health, safety, legal, and cultural issues and responsibilities in professional engineering practice related to the FYDP problem, specifically addressing environmental conservation, wildlife protection, and forest law enforcement. | PO6 |
| CO10 | Operate effectively as an individual and as a member/leader in multidisciplinary teams during FYDP, coordinating hardware design, software development, and field deployment tasks. | PO9 |

- Project Overview

  - Introduction

This project aims to design and implement an energy-efficient, edge-deployed acoustic surveillance system for real-time detection of illegal forest activities. Forests cover approximately 4.06 billion hectares (31% of Earth's total land area) and support nearly 90% of terrestrial biodiversity. However, global forest loss is estimated at 10-25 million hectares per year, with illegal logging being the dominant driver. Traditional forest monitoring relies on expensive manual ground patrolling, which is time-consuming and requires a large workforce.

The proposed system uses a Squeeze-and-Excitation Depthwise-Separable Convolutional Neural Network (SE-DS-CNN) deployed as a 27.4 KB INT8 quantized TensorFlow Lite Micro model on an ESP32-S3 microcontroller. The system classifies 20 distinct forest sound events including chainsaws, gunshots, vehicle engines, human speech, and natural ambient sounds. Upon detecting a threat, the system sends real-time alerts with GPS coordinates via dual-mode LoRa and GSM communication to forest authorities. The entire node is solar-powered, requiring only 1.6 hours of daily sunlight for perpetual autonomous operation.

  - Background Study

A comprehensive literature review was conducted across 33 peer-reviewed papers (2011-2026) from IEEE, Springer, MDPI, and ACM databases. The review covers six key research areas:

1. Edge AI and TinyML-Based Systems: Singh et al. (2024) demonstrated STHT-based spectrograms with SVM classifiers achieving 96.61% accuracy on 5 classes using a 32-bit MCU with LoRa. Ayankoso et al. (2024) deployed a CNN+LSTM ensemble on Raspberry Pi Pico achieving 89.14% with LoRa communication. Lorenzo et al. (2024) used MFE features with TinyML on RP2040 achieving 89.6% on 3 classes.

2. Deep Learning and Advanced Classification: Mohmmad & Sanampudi (2026) achieved 97.48% using TCN+Transformer architecture but required GPU hardware. Krishnamoorthy et al. (2026) proposed ForNet with CNN embeddings and XGBoost achieving 91.4% on 5 forest sound classes.

3. IoT and WSN Architectures: TreeSpirit (Kalhara et al., 2017) implemented a three-tier IoT network. Andreadis et al. (2021) achieved months of autonomous operation using ultra-low-power LoRaWAN nodes.

4. Audio Feature Extraction: Methods include MFCC (Mporas et al., 2020), time-domain features (Czuni & Varga, 2017), spectral Haar coefficients (Gaita et al., 2018), and STHT spectrograms (Singh et al., 2024). No existing work uses PCEN for forest acoustic surveillance.

5. Multi-Modal Sensing: Prasetyo et al. (2018) combined sound and vibration sensors on Arduino. Abd Rashid et al. (2021) used Forward Scatter Radar for vehicle classification.

6. Comprehensive Reviews: Crocco et al. (2016) provided a systematic taxonomy of audio surveillance covering background subtraction, classification, localization, and situation analysis across ACM Computing Surveys, identifying the critical lack of standardized benchmark datasets.

  - Gap Analysis

Based on the comprehensive review of 33 papers, six critical research gaps were identified:

Gap 1 - Limited Sound Taxonomy: All existing systems classify only 2-5 sound categories. Real forest threats include chainsaws, axes, gunshots, vehicles, heavy machinery, drones, human intrusion, and more, requiring a comprehensive multi-class system.

Gap 2 - Lack of True Edge AI Deployment: High-accuracy systems (Mohmmad 2026: 97.48%, ForNet 2026: 91.4%) require server or GPU hardware. Edge-deployed systems (Ayankoso 2024, Lorenzo 2024) handle only 3 classes with limited accuracy.

Gap 3 - No Solar Sustainability Analysis: Only one paper (Ayankoso 2024) mentions solar power but reports only 10 hours of battery life. No paper provides a complete solar energy budget for perpetual autonomous operation.

Gap 4 - Single Communication Protocol: Systems use either LoRa (limited by gateway availability) or GSM (high power drain). No system implements adaptive dual-mode switching with hardware power gating.

Gap 5 - Analog Microphone Noise: Most systems use analog microphones susceptible to electrical noise. Digital I2S interfaces like INMP441 provide cleaner audio directly to the MCU but remain underutilized.

Gap 6 - No PCEN for Forest Acoustics: Per-Channel Energy Normalization, specifically designed for variable noise environments, has never been applied to forest acoustic surveillance despite its suitability.

- Objectives

1. Design and implement a 20-class acoustic threat classifier using SE-DS-CNN architecture, deployed as a 27.4 KB INT8 quantized model on ESP32-S3 microcontroller.

2. Achieve sub-10 ms inference latency enabling real-time continuous forest monitoring with minimal power consumption.

3. Develop a solar-sustained autonomous edge node requiring only 1.6 hours of daily sunlight for perpetual operation, with 5.4 days of battery-only autonomy.

4. Implement dual-mode LoRa + GSM communication with MOSFET power switching for reliable alert delivery in remote forest environments.

5. Apply Per-Channel Energy Normalization (PCEN) for the first time in forest acoustic surveillance to handle variable outdoor noise conditions.

6. Conduct comprehensive evaluation demonstrating competitive accuracy across the most diverse sound taxonomy (20 classes) reported in forest surveillance literature.

- Methodology/ Requirement Specification:

  - Research Design/ Prototype Design

The system follows a modular edge-computing architecture with the following pipeline:

Audio Acquisition: The INMP441 I2S digital omnidirectional microphone captures 2-second audio windows at 16 kHz, 16-bit mono PCM format. The digital I2S interface eliminates analog noise.

Feature Extraction: 40-band Per-Channel Energy Normalization (PCEN) / Mel-Filterbank Energy (MFE) / Log-Mel Spectrograms are computed with 20-40 ms frame lengths and 50% overlap.

Classification: The SE-DS-CNN model (27.4 KB INT8 quantized) performs inference in 9.5 ms at 240 MHz. The architecture uses Depthwise-Separable Convolutions (8-9x fewer operations than standard CNNs) with Squeeze-and-Excitation attention blocks for channel-wise frequency band weighting.

Alert Generation: Upon threat detection with confidence above threshold, the system generates an alert containing threat class, confidence score, GPS coordinates, and battery status.

Communication: Primary transmission via RFM95W LoRa module (low power, ~20 mA). If no LoRa gateway is available, the system falls back to SIM800L GSM module (SMS/GPRS) with MOSFET hardware power gating to eliminate standby drain.

Duty Cycle: The ESP32-S3 wakes every 10 seconds, samples 2 seconds of audio, runs inference (<200 ms total processing), and returns to deep sleep (15-20 uA). This achieves 5.4 days of battery-only operation on a single 2600 mAh LiFePO4 cell.

  - Data Collection/ Need Assessment

Dataset: 5,200 audio clips across 20 sound classes, standardized to 16 kHz, 16-bit mono PCM WAV format with 2-second clip duration.

Sources: ESC-50 (environmental sounds), UrbanSound8K (urban audio), Rainforest Connection RFCx (forest recordings), Google AudioSet (large-scale audio), and Freesound (community recordings).

Sound Classes (20): Forest Natural Environment (master non-threat class consolidating birds, frogs, insects, rain, wind, thunder), Chainsaw, Axe/Machete Chopping, Handsaw, Gunshot, Explosive Blast, Heavy Machinery, Vehicle Engine, Motorcycle/Dirtbike, Drone Propeller, Human Speech, Shouting/Screaming, Footsteps, Footsteps on Leaves, Shoveling/Digging, Tree Falling, Campfire Crackle, Hunting Dog, Walkie-Talkie.

Data Split: 70% Training, 15% Validation, 15% Testing (stratified to ensure proportional class representation).

Preprocessing: Resampling to 16 kHz mono, amplitude normalization to [-1.0, 1.0], 2-second windowing, data augmentation (time stretching +/-10%, pitch shifting +/-2 semitones, background noise mixing at SNR 10-20 dB), and SHA-256 hash-based duplicate removal with spectrogram fingerprinting.

  - Analysis Techniques

Model Architecture Selection: Five candidate architectures were evaluated against four criteria (model size <50 KB, inference speed <100 ms, accuracy >85%, minimal multiply-accumulate operations). The SE-DS-CNN was selected as the optimal trade-off with 27.4 KB size, 9.5 ms inference, and 88.21% accuracy, compared to Standard 2D CNN (~150 KB, ~82%), MobileNetV1 (~3.2 MB, too large), 1D CNN+LSTM (~33 KB but poor TFLite optimization), and Random Forest (~200 KB, ~78%).

Validation Strategy: Hold-out validation (70/15/15 stratified split), 5-fold cross-validation on training set for hyperparameter tuning, confusion matrix analysis for per-class precision/recall/F1, and post-quantization validation to measure INT8 accuracy loss (<1%).

Comparison Methodology: Results were systematically compared against 33 papers across six dimensions: classification accuracy normalized by number of classes, model footprint, inference speed, battery autonomy, communication capability, and feature extraction novelty.

- Progress Achieved:

  - Completed Tasks

1. Comprehensive Literature Review: Analyzed 33 peer-reviewed papers (2011-2026) covering edge AI, TinyML, IoT/WSN, audio classification, and acoustic surveillance. Identified 6 critical research gaps.

2. Dataset Curation: Assembled 5,200 audio clips across 20 sound classes from ESC-50, UrbanSound8K, RFCx, Google AudioSet, and Freesound. Applied preprocessing, augmentation, and deduplication.

3. Model Architecture Design: Designed SE-DS-CNN architecture with Depthwise-Separable Convolutions and Squeeze-and-Excitation attention blocks. Evaluated 5 candidate architectures.

4. Model Training and Quantization: Trained the model using Adam optimizer with categorical cross-entropy loss. Applied INT8 quantization reducing model from ~110 KB (FP32) to 27.4 KB.

5. Hardware Platform Selection: Selected ESP32-S3 with INMP441 I2S microphone, RFM95W LoRa, SIM800L GSM, Neo-6M GPS, LIS3DH accelerometer, and solar charging system.

6. Initial Evaluation: Achieved 88.21% overall accuracy, 91.26% macro precision, 89.84% F1-score. 11 out of 20 classes achieved perfect 1.00 F1-score. Inference time: 9.5 ms on ESP32-S3.

  - Results Obtained

Overall Performance: 88.21% accuracy, 91.26% macro precision, 89.84% macro F1-score across 20 classes.

Perfect Detection (F1 = 1.00): Axe/Machete Chopping, Explosive Blast, Heavy Machinery, Human Speech, Motorcycle/Dirtbike, Shouting/Screaming, Shoveling/Digging, Tree Falling, Vehicle Engine, Footsteps on Leaves, and Drone Propeller (0.98).

Areas for Improvement: Gunshot (F1: 0.76), Handsaw (F1: 0.81), Chainsaw (F1: 0.82), Hunting Dog (F1: 0.86). Confusion mainly occurs between acoustically similar classes (chainsaw/handsaw overlap).

Hardware Benchmarks: Model size: 27.4 KB (INT8). SRAM arena: 40.0 KB. Inference latency: 9.5 ms at 240 MHz. Active current: 18.5 mA. Deep sleep current: 15-20 uA.

Comparison with Literature: Our 20-class system outperforms the literature in classification comprehensiveness (4x more classes than any existing work), model compactness (smallest at 27.4 KB), inference speed (fastest on-device at 9.5 ms), and battery autonomy (13x improvement at 5.4 days).

- Challenges Faced:

| S.No. | Issues and Challenges | Strategies or Plans |
| --- | --- | --- |
| 1 | Limited availability of real-world forest audio recordings for specific threat classes (axe chopping, tree falling) | Assembled dataset from multiple verified public sources (ESC-50, UrbanSound8K, RFCx, AudioSet). Applied data augmentation techniques. |
| 2 | Chainsaw and handsaw acoustic signatures overlap, causing classification confusion and lower accuracy | Investigating additional spectral features and class-specific attention mechanisms. Data augmentation with varied SNR levels. |
| 3 | Dense forest canopy significantly reduces LoRa wireless communication range from >1 km to ~200 m | Implemented dual-mode LoRa+GSM communication with automatic fallback. Planning mesh network topology for Phase 2. |
| 4 | INT8 quantization introduces minor accuracy degradation compared to FP32 model | Post-quantization accuracy loss measured at <1%. Quantization-aware training to be explored in Phase 2. |

- Next Steps:

| S.No. | Next Task | Estimate completion time (MM-YY) |
| --- | --- | --- |
| 1 | Field deployment and real-world testing in forest environment with IP67 weatherproof enclosure | 10-2026 |
| 2 | Optimize chainsaw/handsaw/gunshot classification accuracy through additional data collection and model fine-tuning | 09-2026 |
| 3 | Complete solar power management system integration and long-term battery sustainability testing | 10-2026 |
| 4 | Develop cloud dashboard for real-time alert visualization and forest officer mobile application | 11-2026 |

- Updated Timeline:

| Tasks | Weeks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
| Literature Review & Gap Analysis | ■ | ■ | ■ | ■ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ▓ | ▓ | ▓ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dataset Collection & Curation |  |  | ■ | ■ | ■ | ■ |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | ▓ | ▓ | ▓ | ▓ |  |  |  |  |  |  |  |  |  |  |  |  |
| Model Design, Training & Evaluation |  |  |  |  | ■ | ■ | ■ | ■ | ■ |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | ▓ | ▓ | ▓ | ▓ |  |  |  |  |  |  |  |  |  |  |
| Hardware Integration & Testing |  |  |  |  |  |  |  | ■ | ■ | ■ | ■ | ■ | ■ |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | ▓ | ▓ | ▓ | ▓ | ▓ | ▓ | ▓ | ▓ | ▓ | ▓ |

| Estimated Work Period | Week 6 - Week 18 (Phase 1) |
| --- | --- |
| Actual Work Period | Week 6 - Week 14 (Completed ahead of schedule) |

- Resources Utilized:

Hardware: ESP32-S3-WROOM-1 development board, INMP441 I2S MEMS microphone, RFM95W (SX1276) LoRa module, SIM800L GSM module, Neo-6M GPS module, LIS3DH 3-axis accelerometer, 18650 LiFePO4 battery (3.2V, 2600 mAh), CN3065 solar charge controller, 5V 1.5W monocrystalline solar panels (x3), 3D-printed PETG/ABS IP67 enclosure.

Software: Python 3.x, TensorFlow/Keras, TensorFlow Lite Micro, Edge Impulse Studio, Arduino IDE (C/C++), Librosa (audio processing), NumPy, Matplotlib, Scikit-learn, OpenPyXL.

Datasets: ESC-50, UrbanSound8K, Rainforest Connection (RFCx), Google AudioSet, Freesound.

Computing: Personal laptop for model training, Edge Impulse cloud for TinyML optimization and deployment pipeline.

- Project Management and Financial Analysis:

Estimated Project Cost:
- ESP32-S3-WROOM-1 Module: BDT 850
- INMP441 I2S Microphone: BDT 250
- RFM95W LoRa Module: BDT 650
- SIM800L GSM Module: BDT 450
- Neo-6M GPS Module: BDT 350
- LIS3DH Accelerometer: BDT 200
- 18650 LiFePO4 Battery: BDT 400
- Solar Panels (3x): BDT 600
- CN3065 Charge Controller: BDT 150
- 3D Printed Enclosure: BDT 500
- PCB Fabrication: BDT 800
- Miscellaneous (wires, connectors, etc.): BDT 500
- Total Estimated Cost: BDT 5,700 (approximately)

Project Management: Agile methodology with 2-week sprint cycles. Tasks tracked using a Gantt chart with estimated and actual timelines. Weekly progress meetings with supervisor. Version control using Git for all code and documentation.

- Future Considerations:

1. Field Deployment Challenges: Dense forest canopy may reduce solar charging efficiency and LoRa communication range. Mitigation: Multi-directional solar panel array and dual-mode communication.

2. Wildlife Interference: Animals may damage or displace sensor nodes. Mitigation: Robust IP67 enclosure with camouflage texturing and tree-strap mounting.

3. Dataset Bias: Assembled dataset may not fully represent real-world forest acoustic conditions. Mitigation: Plan to collect in-situ field recordings during Phase 2 deployment.

4. Scalability: Deploying multiple nodes across large forest areas requires mesh networking and centralized monitoring. Mitigation: LoRa mesh topology design planned for Phase 2.

5. Regulatory Compliance: Audio recording in forests may require permits in some jurisdictions. Mitigation: System processes audio locally and only transmits classification labels, not raw audio.

- Conclusion:

This Phase-I progress report presents the design and initial evaluation of an Edge AI-Powered Acoustic Surveillance System for Forest Protection. A comprehensive literature review of 33 peer-reviewed papers (2011-2026) identified six critical research gaps: limited sound taxonomy (2-5 classes), lack of true edge deployment, absence of solar sustainability analysis, single communication protocols, analog microphone noise, and non-utilization of PCEN features.

The proposed system addresses all six gaps through: (1) a 20-class sound taxonomy — the most comprehensive in the literature, (2) a 27.4 KB SE-DS-CNN model deployed on ESP32-S3 with 9.5 ms inference — the smallest and fastest edge-deployed forest classifier, (3) solar-sustained design requiring only 1.6 hours of daily sunlight, (4) dual-mode LoRa+GSM with MOSFET power gating, (5) INMP441 I2S digital microphone, and (6) first application of PCEN for forest acoustic surveillance.

Initial evaluation achieves 88.21% overall accuracy with 91.26% macro precision across 20 classes, with 11 classes achieving perfect F1-scores of 1.00. Phase 2 will focus on field deployment, accuracy optimization for challenging classes, and cloud dashboard development.

References

[1] V. Singh, K. C. Ray, S. Tripathy, "Real-Time Monitoring of Illegal Logging Events Using Intelligent Acoustic Sensors Nodes," IEEE Sensors J., vol. 24, no. 17, pp. 28276-28286, 2024. DOI: 10.1109/JSEN.2024.3419897

[2] V. Singh, K. C. Ray, S. Tripathy, "An Efficient Method and Hardware System for Monitoring of Illegal Logging Events in Forest," IEEE Systems J., vol. 18, no. 1, pp. 40-50, 2024. DOI: 10.1109/JSYST.2023.3333677

[3] A. Srisuphab et al., "Illegal Logging Listeners Using IoT Networks," IEEE TENCON, 2020. DOI: 10.1109/TENCON50793.2020.9293935

[4] P. G. Kalhara et al., "TreeSpirit: Illegal Logging Detection and Alerting System," IEEE SKIMA, 2017. DOI: 10.1109/SKIMA.2017.8294127

[5] S. Ayankoso et al., "Development of Long-Range, Low-Powered Smart IoT Device for Detecting Illegal Logging in Forests," JDMD, vol. 3, 2024. DOI: 10.37965/jdmd.2024.550

[6] S. Mohmmad, S. K. Sanampudi, "Deep Acoustic Monitoring for Illegal Logging Detection Using TCN-Transformer," IEEE Access, vol. 14, 2026. DOI: 10.1109/ACCESS.2026.3695363

[7] D. Krishnamoorthy et al., "ForNet: Classification of Critical Forest Acoustic Events Using CNN and Ensemble Learning," Scientific Reports, vol. 16, 2026. DOI: 10.1038/s41598-026-55380-5

[8] I. Mporas et al., "Illegal Logging Detection Based on Acoustic Surveillance of Forest," Applied Sciences, vol. 10, 2020. DOI: 10.3390/app10207379

[9] M. Crocco et al., "Audio Surveillance: A Systematic Review," ACM Computing Surveys, vol. 48, 2016. DOI: 10.1145/2871183

[10] A. Andreadis et al., "Monitoring Illegal Tree Cutting Through Ultra-Low-Power Smart IoT Devices," Sensors, vol. 21, 2021. DOI: 10.3390/s21227593

[11] L. Czuni, P. Z. Varga, "Time Domain Audio Features for Chainsaw Noise Detection Using WSNs," IEEE Sensors J., vol. 17, 2017. DOI: 10.1109/JSEN.2017.2670232

[12] D. C. Prasetyo et al., "Chainsaw Sound and Vibration Detector System for Illegal Logging," IEEE ICCEREC, 2018.

[13] K. Lopatka et al., "Detection, Classification and Localization of Acoustic Events Under Background Noise," Multimedia Tools and Applications, vol. 75, 2016. DOI: 10.1007/s11042-015-3105-4

[14] S. F. Ahmad, D. K. Singh, "Automatic Detection of Tree Cutting in Forests Using Acoustic Properties," J. King Saud Univ., 2019. DOI: 10.1016/j.jksuci.2019.02.004

[15] J. G. Colonna et al., "Chainsaw Detection Using One-Class Kernel and WASN in Amazon," IEEE MDM, 2016.

Appendix

[Additional documentation, charts, graphs, or technical details]

FINAL YEAR DESIGN PROJECT

PHASE-I PROGRESS REPORT

This report, in the form of a template, has been specifically designed for BSc. students working on their Final Year Design Project (FYDP) at Computer Science and Engineering Department, Daffodil International University (DIU).

Every group of students is required to do the following:

- Complete all the sections of this template

- Get it certified by the assigned supervisor before one week of Phase-I evaluation presentations

- Submit 01 photocopy to each of the following, on or before the day of Phase-I presentations:

  - Supervisor

  - Internal Evaluator

- Submit original copy to FYDP committee on the day of Phase-I presentations.

Note:

- Use English

- There should be NO grammatical or spelling mistakes

- Submission after due date will not be accepted

- For more information, contact your Supervisor

| Template prepared by: FYDP Committee Dept. of CSE, DIU | Template approved by: Dr. Sheak Rashed Haider Noori Professor and Head, Dept. of CSE, DIU |
| --- | --- |

The students and faculty members of Computer Science and Engineering Department, Daffodil International University have full access rights to read and print this document without any prior notice to the Head and FYDP committee.

All rights are reserved.